Definition
A cycloidal drive (or cycloidal speed reducer) is a mechanism used to reduce the speed of an input shaft by a fixed ratio.

It achieves high reduction ratios in a compact size with very low or near-zero backlash.

Commonly used in robotics, automation, and precision motion systems.

In robotics, cycloidal drives are commonly used as RV-type reducers for robot joints (high precision + rigidity + load capacity).

Cycloidal drives behave like a specialised planetary transmission: pin gear acts like an internal gear, cycloidal disc acts like the planet gear, eccentric shaft acts like the carrier/input.

Real-world performance depends heavily on non-ideal effects: clearance (tolerances/assembly/lubrication), deformation (contact compliance), and friction.

Main Components

- Input shaft

- Eccentric bearing (typically cylindrical roller bearing)

- Eccentric shaft / crank (creates eccentricity e)

- Cycloidal disc (lobed profile; often curtate/shortened cycloid)

- Stationary ring gear (pins or rollers)

- Output pins / rollers

- Output disk / carrier plate (captures disc motion and transmits torque)

- Output shaft or carrier bearings (support and transmit output load)

- Housing (stiffness matters for bearing alignment and life)

Theory of Operation
The input shaft drives an eccentrically mounted rolling-element bearing (eccentricity e).

The eccentricity forces the cycloidal disc to “wobble” (orbit) in a small circular path.

As the disc wobbles, it also rotates relative to the housing due to engagement with the stationary ring pins.

The perimeter of the disc engages with a stationary ring gear made of pins/rollers; multiple contacts occur at once (load sharing).

The disc rotates in the opposite direction to the input shaft (typical single-stage configuration).

Output pins pass through oversized holes in the cycloidal disc (pin-holes).

Hole oversize (clearance) is tied to eccentricity and required to convert wobble into smooth output rotation.

This converts the wobbling motion of the disc into steady output shaft rotation via the output disk/carrier.

Radial motion of the disc is not transmitted to the output shaft; the carrier “filters” the orbiting motion into rotation.

In an ideal model, many ring pins contact the cycloidal gear simultaneously; about half may carry load at once depending on geometry/load direction.

In practice, not all pins carry load due to clearances and deformation; which pins carry load changes as the input rotates.

Note:

Operation is conceptually similar to planetary gearing, but uses cycloidal geometry rather than involute teeth.

A major reason cycloidal drives can be efficient is the use of roller pins to shift tooth contact toward rolling friction instead of sliding friction (though sliding still exists in places).

Reduction Ratio

Definitions:

- P = number of pins in the ring gear

- L = number of lobes on the cycloidal disc

Reduction ratio:

r = (P − L) / L

Notes:

- Ring gear always has more pins than disc lobes.

- A small difference between P and L produces a large reduction ratio.

- Common “classic” arrangement is P = L + 1 (one-tooth difference), which yields high reduction in a single stage.

Two common product configurations exist (depends what is fixed and what is output):

- Configuration A: ring/pin gear fixed, output disk outputs

- Configuration B: output disk fixed, pin gear outputs

- These configurations swap what you treat as output, but the underlying relative motion and ratio relationships remain linked to pin and lobe counts.

In real drives, instantaneous transmission ratio can fluctuate due to clearance and deformation (ratio ripple), which can appear as disturbance torque in robot control loops.

Efficiency

- Single-stage cycloidal drives: approximately 93% efficiency (typical quoted figure).

- Double-stage cycloidal drives: approximately 86% efficiency (typical quoted figure).

- Commercially available reduction ratios:

- Single stage: up to approximately 119:1

- Double stage: up to approximately 7,569:1

- Ideal “catalogue” efficiencies assume good lubrication and manufacturing; real efficiency depends strongly on friction at specific contacts.

- Efficiency is harder to estimate accurately than stiffness/backlash because it depends on contact forces + sliding velocities + friction coefficients across multiple contacts.

- Numerical methods (like CDFM in Qi et al., 2024) are used because simplified formulas often miss how design parameters change the friction losses.

- Key efficiency insight (Qi et al., 2024): increasing reduction ratio (via higher tooth/pin count relationships) can significantly reduce efficiency under the same friction and load conditions.

- Key efficiency insight (Qi et al., 2024): friction at the eccentric shaft / input shaft interface tends to dominate the overall efficiency loss compared with other contact pairs.

Cycloidal Disc Design

- Cycloidal discs are often designed using a shortened (curtate) cycloid profile.

- Reduces eccentricity required for a given design

- Minimises unbalance forces

- Improves high-speed performance

- Common practice is to use two cycloidal discs offset by 180 degrees.

- Cancels static imbalance

- Can be used to reduce backlash when preloaded appropriately

- Some high-speed designs use three or more discs.

- Outer discs move together

- Middle disc is heavier and moves in opposition

- Further reduces vibration (dynamic balancing improvement)

- Clearance is not purely “bad”; it is intentionally designed in to account for:

- Manufacturing tolerances and geometric deviations

- Thermal expansion/deformation

- Lubrication film requirements

- Assembly misalignment

- Qi et al. (2024) highlights that small geometric deviations (e.g., eccentricity deviation Δe) change:

- Which pins contact under load

- Peak contact forces and contact stresses

- Bearing loads on the eccentric shaft

- Backlash (lost motion) and stiffness

- Overall efficiency

- Positive vs negative eccentricity deviations are not equivalent; they can trade off backlash/accuracy vs load capacity/efficiency depending on how contact positions shift.

Advantages

- Very low or zero backlash (especially with dual-disc preload designs)

- High torque capacity relative to size

- Compact packaging compared to involute gearboxes

- High shock load resistance (force shared across many contacts)

- Large contact area

- Force shared across many teeth/pins simultaneously

- High stiffness potential under load due to multiple contact points (though clearances reduce stiffness at low load)

- Suitable for low-speed, high-torque applications

- Well suited to robot joints (RV reducers) where stiffness and load capacity matter

- Load sharing means individual pin forces can be lower than a comparable single-mesh gear system (ideal case)

Disadvantages

- Eccentric motion introduces vibration.

- Single-disc designs produce significant vibration if unbalanced.

- Increased wear on disc teeth and bearings if poorly balanced.

- Two-disc designs remove static imbalance but retain small dynamic imbalance.

- High-speed applications may require three or more discs for balance.

- Sliding contact reduces efficiency and increases lubrication requirements (even with roller pins, not everything is pure rolling).

- More complex geometry than involute gears (harder to design, manufacture, and tolerance properly).

- Backlash/lost motion and stiffness are highly sensitive to clearances and geometric deviations; performance can vary strongly with tolerances and assembly.

- Eccentric shaft and input shaft bearing loads can become large; poor design here can be a primary failure mode and a major efficiency loss source (Qi et al., 2024).